Question 1

Assume the following instruction trace. ADD/SUB operations take one cycle, MUL operations take three cycles, and Load/Store operations take two cycles. The pipeline has 2 ADD/SUB units, 1 MUL unit, and 1 Load/Store unit. The functional units are pipelined.

- add R1, R2, R2

- mul R3, R1, R3

- lw    R2, 0(R1)

- sub R4, R3, R2

- mul R6, R5, R5

- add R7, R6, R2

- add R7, R4, R7

- sw R7, 0(R8)

- Draw the dependence graph for the 8 instructions shown above. Include all anti, output, and pure dependencies with arcs.

- Show the dependence height of each instruction (multiple answers possible).

| INST | height |
| --- | --- |
| A | 8 |
| B | 7 |
| E | 7 |
| C | 6 |
| D | 4 |
| F | 4 |
| G | 3 |
| H | 2 |

- Show the best VLIW schedule you can find. You may show the schedule using letters only.

| ALU 1 |  |  |  | ALU 2 |  |  |  | MUL |  |  |  | LOAD / STORE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INST | DEST | SRC1 | SRC2 | INST | DEST | SRC1 | SRC2 | INST | DEST | SRC1 | SRC2 | INST | D / S | SRC |
| A | R1 | R2 | R2 |  |  |  |  | E | R6 | R5 | R5 |  |  |  |
|  |  |  |  |  |  |  |  | B | R3 | R1 | R3 | C | R2 | R1 |
| D | R4 | R3 | R2 |  |  |  |  |  |  |  |  |  |  |  |
| F | R7 | R6 | R2 |  |  |  |  |  |  |  |  |  |  |  |
| G | R7 | R4 | R7 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | H | R7 | R8 |

- Based on your result from c), please generate an encoding scheme for the instructions using the tail and pause bits scheme.

| INST | OP | DEST | SRC 1 | SRC 2 | TAIL | PAUSE |
| --- | --- | --- | --- | --- | --- | --- |
| A | ADD | R1 | R2 | R2 | 0 | 0 |
| E | MUL | R6 | R5 | R5 | 1 | 0 |
| B | MUL | R3 | R1 | R3 | 0 | 0 |
| C | LW | R2 | R1 | - | 1 | 0 |
| D | SUB | R4 | R6 | R2 | 1 | 1 |
| F | ADD | R7 | R6 | R2 | 1 | 0 |
| G | ADD | R7 | R4 | R7 | 1 | 0 |
| H | SW | R7 | R8 | - | 1 | 0 |

Question 2

Consider the following loop. (Although it's not relevant to the solution, you can assume that R3 begins with a negative value stored in it.)

LABEL:A: ADD R1, R1, #1

B: MUL R1, R1, 4

C: LW    R2, 100(R1)

D: SW    R2, 200(R1) // Note: R2 is a SOURCE register for SW

E: ADD R3, R3, #1

F: BRNEZ R3, LABEL // branch to LABEL if R3 != 0

Referring to Software Pipelining via Iterative Modulo Scheduling, what is the RecMII and ResMII for this loop? What is the overall minimum II? Assume there is 1 ALU unit, 1 MUL unit, 1 LW unit and 1 SW unit. Both MUL and LW takes 2 cycles and are pipelined. SW, ADD and BREZ all take 1 cycle. ADD and BREZ use the ALU unit. Now show a prologue, kernel and epilog after modulo scheduling. For simplicity, you can write the letter of the operation in a table. You do not need to write the instructions themselves.